There are various terms in use when discussing high voltage vehicles. The table below explains the key aspects of this terminology.

| Term | Explanation |
| --- | --- |
| Vehicle High Voltage | DC Voltage above 60 Volts and AC Voltage above 25 V. |
| High Energy | Voltage and or current levels that exceed the safe working thresholds within a vehicle of 25 V AC, 60V DC and 16mA. |
| Hazardous Voltage | Voltage and or current levels that exceed the safe working thresholds within a vehicle of 25 V AC, 60V DC and 16mA. |
| Vehicle Low Voltage | Voltage and or current levels that DO NOT exceed the safe working thresholds of 25 V AC, 60V DC and 16mA. |
| Domestic Low Voltage | British Standard BS 7671:2008 defines supply system low voltage as: 50 to 1000 VAC or 120 to 1500 V ripple-free DC between conductors; 50 to 600 VAC or 120 to 900 V ripple-free DC between conductors and Earth. |
| Commercial High Voltage | The International Electrotechnical Commission and its national counterparts (IET, IEEE, VDE, etc.) define high voltage as above 1000 V for alternating current, and at least 1500 V for direct current—and distinguish it from low voltage (50 to 1000 VAC or 120–1500 VDC) and extra low voltage (<50 VAC or <120 VDC) circuits. This is in the context of building wiring and the safety of electrical apparatus. |